# APPLICATION DETAILS - NEW CONSTRUCTION SECTION VIEWS

STONEDEKS SYSTEM

14600 COMMERCE STREET NE ALLIANCE, OHIO 44601

PHONE: (330) 821-1585

www.stonedeks.com

|  | EXISTING STRUCTURE |
| --- | --- |
| PAVER (OR MANUFACTURED STONE) |  |
| SILCA GRATE |  |
| JOISTS | ATTACHMENT TO EXISTING |
| BEAM PER TABLES | STRUCTURE PER LOCAL CODE |
| UP TO 12" CANTILEVER |  |

1'

GRADELOCALDEPART MENT

GRADE

JOIST SPAN "L"

6X6 NO.2 SYP TREATED COLUMN, UP TO 10' ABOVE GRADE

CONCRETE FOOTING PADS PER TABLES

BELOWP

DEPTH BUI LDING

THIC

DIAMETER

NEW CONSTRUCTION SECTION

MAXIMUM JOIST SPANS "L" - WHEN USING PAVERS 3/4" TO 3" THICKNESS

|  | NATURAL STONE OR MANUFACTURED CONCRETE PAVER THICKNESS |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| JOIST SIZE | 3/4" | 1 1/4" | 2" | 2 1/2" | 3" |
| 2X6 | 8'-9" | 8'- 5" | 7'-10" | 7'-5" | 7'-2" |
| 2X8 | 11'- 4" | 10'- 9" | 10'-0" | 9'-6" | 9'-2" |
| 2X10 | 13'- 7" | 12'- 10" | 11'- 11" | 11'-4" | 10'- 11" |
| 2X12 | 16'- 0" | 15'- 2" | 14'-1" | 13'-4" | 12'- 10" |

NOTES:

- INSTALLATION TO BE COMPLETED IN ACCORDANCE WITH MANUFACTURER'S SPECIFICATIONS.

- DO NOT SCALE DRAWING.

- THIS DRAWING IS INTENDED FOR USE BY ARCHITECTS, ENGINEERS, CONTRACTORS, CONSULTANTS AND DESIGN PROFESSIONALS FOR PLANNING PURPOSES ONLY. THIS DRAWING MAY NOT BE USED FOR CONSTRUCTION.

- FOR PRODUCT AND COMPANY INFORMATION VISIT WWW.STONEDEKS.COM OR CALL

330-821-1585.

StoneDeksTM- revised 2.2018